LC-MS Orbitrap data processing using CoreMS

Notes on data acquisition

SPE extracts were received and analyzed on the Orbitrap by Ioana Bociu in April 2024.

- Samples C24 and C39 were missing and so were not run on the Orbitrap.

- Prior to analysis, samples were reconstituted in 50/50 MeOH/Acetonitrile (Need to confirm this ratio with Ioana – she mentioned that acetonitrile was added due to a high-pressure issue and a chromatogram hump observed when MeOH-only samples were injected).

- Two process blank samples (filter and solution) + 48 samples = 50 in total

- The two filter and solution process blanks were run only once (no technical replicates), and each sample was run in triplicate (three technical reps).

- Total number of raw files per mode = 48 samples * 3 replicates + 2 blanks = 146

Data processing

Raw files were first processed using CoreMS and then further processed in R, following the general protocol (Pakkir Shah, Abzer K., et al. "Statistical analysis of feature-based molecular networking results from non-targeted metabolomics data." Nature protocols 20.1 (2025): 92-162.), by Jieun Kim in April 2026. The processing details are as follows:

- CoreMS processing

The following CoreMS parameters were used for both positive and negative modes (The parameters CoreMS requires are different from formularity processing, so standard values used in Sequim were adopted):

- LC block size: 0.5 min intervals from 0.5 to 10.5 min RT

- Noise threshold: 10

- Molecular search tolerance:  0.5 ppm

- Elemental constraints: C1-100 H4-200 O1-23 N0-2 S0-1

Note that P was not included in the formula search as recommended (including additional elements can increase incorrect formula assignments); however, P can be included if desired. Similarly, the S range can be adjusted to be 0-2 instead of 0-1, but this may also increase incorrect assignments.

- Blank and replicate correction using R

The output csv files were imported into R. POS and NEG runs were processed separately, using the following filtering steps:

- Removed any features present in the process blanks.

- Only retained the features found in 2/3 of technical replicates.

- Only retained the features appeared in ALL (5/5) analytical replicates – this filter reduced the # of features by an order of magnitude.

Note:

Positive mode runs showed extremely high noise levels (abnormally large number of detected/assigned features – a lot of these were removed by the filtering steps)